# 1. Trusted Sources Reviewed

Sources span three kinds of authority: (a) binding U.S. regulation (FDA), (b) international consensus standards (ISO, IEC), and (c) decades-proven operational practice already used by hospital clinical engineering departments to decide which equipment gets priority maintenance — independent of, and pre-dating, any cybersecurity framing.

[1] eCFR / U.S. FDA, 21 CFR Part 860 — Medical Device Classification Procedures, §860.3 (Definitions), Code of Federal Regulations.

https://www.ecfr.gov/current/title-21/chapter-I/subchapter-H/part-860

Relevance: The legal basis for U.S. medical device risk classes. Class III = life-supporting, life-sustaining, or substantial-importance-to-health devices, or those with unreasonable risk of illness/injury. Class II = moderate risk, needs special controls. Class I = general controls sufficient, not life-supporting/sustaining. This is the single legally-authoritative anchor already used as CAS's primary CC signal.

[2] International Organization for Standardization, ISO 14971:2019 — Medical devices — Application of Risk Management to Medical Devices, 2019.

https://www.iso.org/standard/72704.html

Relevance: The international standard for medical-device risk analysis. Defines severity of harm on a scale from Negligible to Catastrophic and requires risk to be evaluated as probability × severity, not severity alone. Supplies the ‘how bad if this fails’ axis that FDA class alone does not fully separate within Class II.

[3] MITRE Corporation, prepared for the U.S. Food and Drug Administration, “Rubric for Applying CVSS to Medical Devices,” 2019 (FDA-qualified Medical Device Development Tool, Oct. 2020).

https://www.mitre.org/news-insights/publication/rubric-applying-cvss-medical-devices

Relevance: Its Environmental (Safety) worksheet walks an analyst through a structured clinical-impact classification per device — reusable directly as a cross-check questionnaire when assigning a new device to a CC tier.

# 2. The Five-Tier Hierarchy

This hierarchy maps directly onto CAS's CC scale (1–5). FDA device class is used as the primary anchor because it is the only criterion here with legal force and full coverage of every device; the other five criteria refine placement within a class and resolve borderline cases.

## Tier 5  (CC = 5.0, FDA Class III) — Life-Sustaining / Critical Care

Direct, continuous, real-time control of a vital physiological function. Compromise can cause death or catastrophic, immediate harm with little or no time to intervene.

Examples: ICU ventilators, defibrillators/ICD, pacemaker programmers, neonatal incubators, closed-loop insulin pumps, ECMO, haemodialysis machines.

Criteria support: C1=III, C2=Therapeutic/life-support (max), C3=Catastrophic, C4=direct/essential, C5=often no backup mid-procedure, C6=single-patient but fatal

## Tier 4  (CC = 4.0, FDA Class II) — Direct Patient Monitoring / Diagnostic-Therapeutic

Continuously attached to or imaging a specific patient; failure delays detection of a life-threatening event or a procedure, but the device is not itself sustaining life moment-to-moment.

Examples: Bedside multiparameter monitors, cardiac/ECG monitors, MRI/CT scanners, central monitoring stations.

Criteria support: C1=II, C2=Diagnostic/monitoring, C3=Serious–Critical, C4=direct but not essential-performance-critical every second, C5=usually has staff observation as backup

## Tier 3  (CC = 3.0, FDA Class II / I) — Clinical Data & Informatics Systems

Handles clinical data, images, or records that inform care indirectly; a fault causes delay or misinformation rather than an immediate physical event.

Examples: PACS/imaging workstations, EHR/clinical web portals, clinical database servers, laboratory information systems, point-of-care analysers.

Criteria support: C1=II (data-integrity-driven) or I, C2=Diagnostic-support/Analytical, C3=Minor–Serious (indirect harm via misdiagnosis/delay), C4=one remove from the patient, C6=can be ward- or hospital-wide (shared record system)

## Tier 2  (CC = 2.0, FDA Class I) — Clinical IT Infrastructure & Operational Support

Supports clinical operations but has no direct diagnostic or therapeutic function of its own.

Examples: Nurse workstations/COWs, clinical PCs (RDP), hospital bed-management systems, internal DNS/network infrastructure.

Criteria support: C1=I, C2=Analytical/support (low), C3=Minor, C6=can be hospital-wide if shared infrastructure (e.g. DNS) despite low per-device risk

## Tier 1  (CC = 1.0, FDA Class I) — Administrative / Non-Clinical

No patient-care role at all. Under Fennigkoh & Smith this tier typically would not score high enough (EM ≤ 12) to enter an active clinical equipment-management program in the first place.

Examples: Admin/clerical workstations, network printers.

Criteria support: C1=I, C2=Administrative (lowest), C3=Negligible, C6=single-device, no patient link

# 5. Validation Against MediSIEM's Own Device Table

CAS's live cas_config.json / DEVICE_PROFILES table already assigns 91 real medical and hospital-IT device types to a CC value via FDA device class (see cas_engine.py's own docstring). The sample below — spanning all five tiers — checks that each assignment is independently consistent with the criteria in Section 3, not just with FDA class alone.

| Device (from DEVICE_PROFILES) | FDA Class | CC | Department | Supporting Criteria |
| --- | --- | --- | --- | --- |
| ICU Mechanical Ventilator | III | 5.0 | ICU | C1, C2, C3, C4 |
| Defibrillator / ICD Network | III | 5.0 | Emergency / ICU | C1, C2, C3, C4 |
| Pacemaker / ICD Programmer | III | 5.0 | Cardiology | C1, C2, C3, C4 |
| Neonatal Incubator / Warmer | III | 5.0 | NICU | C1, C2, C3, C5 (vulnerable patient, thermal risk) |
| Networked Insulin Pump (Closed-Loop) | III | 5.0 | Endocrinology / ICU | C1, C2, C3, C4 (NIST SP1800-8 precedent) |
| Bedside Patient Monitor (Multiparameter) | II | 4.0 | ICU / General Ward | C1, C2, C3 |
| Cardiac Monitor / 12-Lead ECG | II | 4.0 | Cardiology / ICU | C1, C2, C3 |
| MRI Scanner (Control System) | II | 4.0 | Radiology | C1, C2, C3 (imaging/physical hazard) |
| CT Scanner (Control / DICOM Interface) | II | 4.0 | Radiology | C1, C2, C3 |
| Central Patient Monitoring Station | II | 4.0 | ICU / Step-Down | C1, C2, C6 (aggregates multiple patients) |
| DICOM PACS / Imaging Workstation | II | 3.0 | Radiology | C1, C2 (informatics), C3 (indirect harm) |
| Clinical Web Portal / EHR | II | 3.0 | Clinical Informatics | C1, C2, C6 (hospital-wide record access) |
| Clinical Database Server (PostgreSQL) | II | 3.0 | Clinical Informatics | C1, C6 |
| Laboratory Information System (LIS) | II | 3.0 | Laboratory | C1, C2, C3 |
| Point-of-Care Lab Analyser | II | 3.0 | Laboratory / ICU | C1, C2, C3 |
| Nurse Workstation / COW | I | 2.0 | All Wards | C1, C2 (support role) |
| RDP Workstation / Clinical PC | I | 2.0 | Administration / Clinical IT | C1, C2 |
| Hospital Bed Management System | I | 2.0 | Administration | C1, C6 (logistics, hospital-wide) |
| DNS Server (Hospital Network) | I | 2.0 | IT Infrastructure | C1, C6 (shared infra, wide blast radius) |
| Admin / Clerical Workstation | I | 1.0 | Administration | C1, C2, C3 (negligible) |
| Network Label / Results Printer | I | 1.0 | Administration | C1, C2, C3 (negligible) |

# 6. Classifying a New or Unrecognised Device

When a device's port/protocol isn't in the 91-entry table, cas_engine.py falls back to a conservative default (FDA Class I, CC = 1.0). To assign a real tier instead of relying on the default, work through the checklist in order:

Look up the device's FDA listing (or manufacturer's stated intended use) for its 21 CFR §860.3 class — this alone usually narrows it to one or two adjacent tiers (C1).

Ask: if this device stopped working or gave wrong output right now, could that alone kill or seriously injure a patient within minutes? If yes → Tier 5. If it would delay or degrade care but a clinician would likely notice and intervene → Tier 4 (C2, C3, C4).

If the device holds or moves clinical data rather than acting on the patient's body → Tier 3 (C2, C4).

If it supports clinical operations but has no diagnostic/therapeutic function → Tier 2 (C2).

If it has no patient-care role at all → Tier 1 (C2, C3).

Sanity-check against redundancy and blast radius (C5, C6): a Tier-3/4 device with no backup and hospital-wide reach (e.g. a shared imaging server) may warrant rounding up one tier.